LAB

REPORT

IRE 212 :  IoT Architecture and Technologies Sessional

| PREPARED BY Mehrin Farzana ID: 2101013 Session: 2021-2022 Date: 4/12/2024 | SUPERVISED BY Suman Saha Assistant Professor & Chairman Department of IRE, BDU |
| --- | --- |

|  | BANGABANDHU SHEIKH MUJIBUR RAHMAN DIGITAL UNIVERSITY (BDU) |
| --- | --- |

Lab Report-11

Experiment Title: GPS Module Interfacing With Arduino UNO.

# Theory:

# GPS (Global Positioning System) is a satellite-based navigation system that provides precise location, altitude, and time data. It consists of a network of satellites that transmit signals to GPS receivers. By analysing these signals, a receiver can calculate its exact position on Earth.

# The GPS module used in this experiment receives signals from satellites and sends raw NMEA (National Marine Electronics Association) data to the Arduino via a serial connection. The TinyGPS++ library is employed to decode this data into useful information such as latitude, longitude, altitude, and time.

# Key Functions of GPS:

- Location Tracking: Determining latitude and longitude coordinates.

- Altitude Measurement: Providing height information above sea level.

- Time Data: Receiving UTC time from satellites.

# Components:

# Components Required for this Project are:

- Arduino UNO: The microcontroller board used for interfacing and data processing.

- GPS Module (e.g., NEO-6M): Receives signals from satellites and provides NMEA data.

- TinyGPS++ Library: Parses and extracts meaningful data from raw GPS data.

- USB Cable: For powering the Arduino and monitoring serial output.

- Connecting Wires: To establish a connection between the Arduino and GPS module.

- Computer with Arduino IDE: Used for programming and viewing the output.

# Circuit Diagram:

# Fig-1: GPS Module Interfacing With Arduino UNO

# Code:

#include <TinyGPS++.h>

#include <SoftwareSerial.h>

/* Create object named bt of the class SoftwareSerial */

SoftwareSerial GPS_SoftSerial(4, 3);/* (Rx, Tx) */

/* Create an object named gps of the class TinyGPSPlus */

TinyGPSPlus gps;

volatile float minutes, seconds;

volatile int degree, secs, mins;

void setup() {

Serial.begin(9600); /* Define baud rate for serial communication */

GPS_SoftSerial.begin(9600); /* Define baud rate for software serial communication */

}

void loop() {

smartDelay(1000); /* Generate precise delay of 1ms */

unsigned long start;

double lat_val, lng_val, alt_m_val;

uint8_t hr_val, min_val, sec_val;

bool loc_valid, alt_valid, time_valid;

lat_val = gps.location.lat(); /* Get latitude data */

loc_valid = gps.location.isValid(); /* Check if valid location data is available */

lng_val = gps.location.lng(); /* Get longtitude data */

alt_m_val = gps.altitude.meters();  /* Get altitude data in meters */

alt_valid = gps.altitude.isValid(); /* Check if valid altitude data is available */

hr_val = gps.time.hour(); /* Get hour */

min_val = gps.time.minute();  /* Get minutes */

sec_val = gps.time.second();  /* Get seconds */

time_valid = gps.time.isValid();  /* Check if valid time data is available */

if (!loc_valid)

{

Serial.print("Latitude : ");

Serial.println("*****");

Serial.print("Longitude : ");

Serial.println("*****");

}

else

{

DegMinSec(lat_val);

Serial.print("Latitude in Decimal Degrees : ");

Serial.println(lat_val, 6);

Serial.print("Latitude in Degrees Minutes Seconds : ");

Serial.print(degree);

Serial.print("\t");

Serial.print(mins);

Serial.print("\t");

Serial.println(secs);

DegMinSec(lng_val); /* Convert the decimal degree value into degrees minutes seconds form */

Serial.print("Longitude in Decimal Degrees : ");

Serial.println(lng_val, 6);

Serial.print("Longitude in Degrees Minutes Seconds : ");

Serial.print(degree);

Serial.print("\t");

Serial.print(mins);

Serial.print("\t");

Serial.println(secs);

}

if (!alt_valid)

{

Serial.print("Altitude : ");

Serial.println("*****");

}

else

{

Serial.print("Altitude : ");

Serial.println(alt_m_val, 6);

}

if (!time_valid)

{

Serial.print("Time : ");

Serial.println("*****");

}

else

{

char time_string[32];

sprintf(time_string, "Time : %02d/%02d/%02d \n", hr_val, min_val, sec_val);

Serial.print(time_string);

}

}

static void smartDelay(unsigned long ms)

{

unsigned long start = millis();

do

{

while (GPS_SoftSerial.available())  /* Encode data read from GPS while data is available on serial port */

gps.encode(GPS_SoftSerial.read());

/* Encode basically is used to parse the string received by the GPS and to store it in a buffer so that information can be extracted from it */

} while (millis() - start < ms);

}

void DegMinSec( double tot_val)   /* Convert data in decimal degrees into degrees minutes seconds form */

{

degree = (int)tot_val;

minutes = tot_val - degree;

seconds = 60 * minutes;

minutes = (int)seconds;

mins = (int)minutes;

seconds = seconds - minutes;

seconds = 60 * seconds;

secs = (int)seconds;

}

# Explanation of Code:

# Initialization:

# Libraries:

# The TinyGPS++ library handles parsing and extracting meaningful GPS data from raw NMEA strings received from the GPS module.

# The SoftwareSerial library enables serial communication on arbitrary digital pins for GPS data transmission.

# Objects and Pins:

# GPS_SoftSerial is initialized for software serial communication on pins 4 (Rx) and 3 (Tx).

# gps is the object used to decode GPS data.

# Data Processing in the Loop:

# The loop() function continuously fetches and processes data from the GPS module.

# Retrieve and Validate Data:

# The gps.location.lat() and gps.location.lng() methods retrieve latitude and longitude in decimal degrees, and their validity is checked.Altitude and time are similarly extracted and validated.

# Invalid Data Handling:

# For invalid data (e.g., no GPS signal), placeholders ("*****") are displayed for latitude, longitude, altitude, or time.

# Output:

Fig-2: Serial Monitor reading of GPS Module Interfacing

Experiment Title: GPS Module Interfacing With ESP32

# Circuit Diagram:

Fig-3: GPS Module Interfacing With ESP32

# Code:

#include <WiFi.h>

#include <TinyGPS++.h>

// GPS Pins

#define RXPin 16

#define TXPin 17

HardwareSerial GPS(1); // Use Serial1 for GPS

// GPS Library

TinyGPSPlus gps;

// Wi-Fi Credentials

const char* ssid = "HCN";

const char* password = "mehedi3311";

// Server Details

const char* server = "http://example.com"; // Replace with your server's endpoint

void setup() {

// Initialize Serial Monitor

Serial.begin(115200);

delay(100);

// Initialize GPS Serial

GPS.begin(9600, SERIAL_8N1, RXPin, TXPin);

// Connect to Wi-Fi

WiFi.begin(ssid, password);

Serial.print("Connecting to Wi-Fi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWi-Fi Connected!");

}

void loop() {

while (GPS.available()) {

if (gps.encode(GPS.read())) { // Process GPS data

if (gps.location.isUpdated()) {

// Get Latitude and Longitude

String latitude = String(gps.location.lat(), 6);

String longitude = String(gps.location.lng(), 6);

Serial.print("Latitude: ");

Serial.println(latitude);

Serial.print("Longitude: ");

Serial.println(longitude);

// Send data to the server

sendToServer(latitude, longitude);

}

}

}

}

void sendToServer(String latitude, String longitude) {

if (WiFi.status() == WL_CONNECTED) {

WiFiClient client;

if (client.connect(server, 80)) {

// Prepare data to send

String postData = "latitude=" + latitude + "&longitude=" + longitude;

// HTTP POST Request

client.println("POST /gpsdata HTTP/1.1");

client.println("Host: example.com");

client.println("Content-Type: application/x-www-form-urlencoded");

client.print("Content-Length: ");

client.println(postData.length());

client.println();

client.println(postData);

Serial.println("Data sent to server: " + postData);

} else {

Serial.println("Failed to connect to server");

}

} else {

Serial.println("Wi-Fi not connected");

}

}

# Output:

Fig-4: Serial Monitor reading of GPS Module Interfacing